Pin Name Changes from IRS3B to IRS3D

| Pin # | Pin Name | Net Name | → | New Name | New Net |
| --- | --- | --- | --- | --- | --- |
| 1 | TRGthresh1 | TRGthresh_1 | → | 1 | VDD1 |
| 5 | TRGthresh2 | TRGthresh_2 | → | 5 | GND5 |
| 9 | TRGthresh3 | TRGthresh_3 | → | 9 | VDD9 |
| 13 | TRGthresh4 | TRGthresh_4 | → | 13 | GND13 |
| 17 | TRGthresh5 | TRGthresh_5 | → | 17 | VDD17 |
| 21 | TRGthresh6 | TRGthresh_6 | → | 21 | GND21 |
| 25 | TRGthresh7 | TRGthresh_7 | → | 25 | VDD25 |
| 29 | TRGthresh8 | TRGthresh_8 | → | 29 | GND29 |
| 35 | Wbias | Wbias | → | Shin | eShin |
| 63 | RD_ENA | eRD_ENA | → | GCC_Reset | eGCC_Reset |
| 65 | 65 | GND65 | → | GCC_CLKn | GCC_CLKn |
| 66 | CLR | eCLR | → | GCC_CLKp | GCC_CLKp |
| 67 | Start | eStart | → | 67 | GND67 |
| 94 | TSTout | eTSTout | → | DONE-In | eDONE-In |
| 95 | Vdly | Vdly | → | DONE | eDONE |
| 101 | 101 | GND101 | → | 101 | VDD101 |
| 102 | 102 | VDD102 | → | CMP_VSS | CMP_VSS |
| 105 | 105 | VDD105 | → | 105 | GND105 |
| 107 | 107 | GND107 | → | Vdischarge | Vdischarge |
| 119 | eTRGMON | TRGMON | → | eRD_ENA | RD_ENA |
| 120 | eRCO_SSXmon | RCO_SSXmon | → | eRCO_SSXmon | Timing2 |
| 121 | eMonTiming | MonTiming | → | eMonTiming | Timing1 |
| 126 | eSSTin | SSTin | → | 126 | GND126 |
| 127 | 127 | GND127 | → | SSTinN | SSTinN |
| 128 | 128 | VDD128 | → | SSTinP | SSTinP |

Other Changes

- Removed 8-bit DAC

- Removed I2C shift registers (2) and associated PUResistors

- Removed all temp sensors except for 1 (ASIC1)

- Changed amplifier resistors to 50 & 20K, similar to RevD design, amplifiers will also be changed but footprint remains the same

- DO NOT INSTALL shunt resistor from pin 110 (ISEL)

- set up voltage divider for pin 127 (SSTinN) and 100nF filter cap

- set up voltage divider for pin 65 (GCC_CLKN) and 100nF filter cap

- Repurposed nets for SSTinP & GCC_CLKp

- ASIC schematic (Logic) re-organized for IRS3D4

- PCA9534A GPIO has pin 11 floating (on top page); this is ok from data sheet

- Similar changes made to Carrier_13 RevE